# Reprocessing of Reusable Equipment in Aged Care Guide

Reprocessing of medical equipment in aged care is crucial for infection prevention and resident safety.

# 1. Key points include:

- Reprocessing level depends on infection risk based on item use.

- Most reusable equipment in aged care is non-critical.

- Critical and semi-critical items are typically disposable.

- Reprocess after each use and on a routine schedule.

- Follow manufacturer's instructions for compatibility and methods.

- Visually inspect equipment for cleanliness and functionality before storage or use.

# 2. Best practices

- Train health care workers (HCW) involved in reprocessing.

- Establish a routine cleaning schedule.

- Use appropriate personal protective equipment.

- Maintain documentation for quality assurance – as appropriate.

# 3. Template

Heat and chemical:

| Sanitizer – For bedpan and urinal use only., Heat (hot water) sanitization only. | Disinfectors Washer -For all reusable items: bedpans, urinals, tooth mugs, bowls. Heat (hot water) and detergent sanitization. |
| --- | --- |

Cold chemical:

Cleaning frequency may increase during outbreak

| Reusable shared Equipment | Frequency | Procedure |
| --- | --- | --- |
| Shower chair This Photo by Unknown Author is licensed under CC BY-NC | After each use, and daily (weekly if single user) | - Use detergent wipes/disinfectant wipes - Wipe base and back of chair - Wipe underside of seat - Wipe chair legs Include in routine cleaning schedule. |
| Bedpans, urinals, urine jug | After each use | - Bedpans and urinals clean and soiled must be covered with a paper cover when being transported - Contents of the bedpan or urinal must be emptied into the toilet at the point of care and not transported through the home - Place bedpan/commode pan/urinal directly into the disinfectant washer after use. - Used items should not be stacked and left on the top of the disinfectant washer |
| Observation machine and Blood Pressure Cuff | After each use | - Check manufacturer’s instruction. - Use detergent wipes/disinfectant wipes - Wipe over all parts of the machine and cuff Include in routine cleaning schedule. |
| Bowls Washing | After each use | - Use Washer Disinfector - Where there is no machine: - HCW to wear PPE -apron and gloves - Clean bowl with warm water and detergent after each use and rinse. - Store bowls separately and inverted to reduce contamination |
| Commodes This Photo by Unknown Author is licensed under CC BY-ND | After each use, and daily | - Use detergent wipes/disinfectant wipes - Wipe base and back and armrests of chair - Wipe underside of seat Wipe chair wheels and foot plates - Ensure, pan is clean post use Include in routine cleaning schedule. |
| Glucometer This Photo by Unknown Author is licensed under CC BY-NC-ND | After each use | - Ideally older person has their own device - Use detergent wipes/disinfectant wipes - Check manufacturer’s instruction. - Thoroughly wipe over all parts |
| Hoists | After each use and daily | - Use detergent wipes/disinfectant wipes - Wipe over the frames - Wipe handlebars and grips - Wipe base and wheels - Wipe controls Include in routine cleaning schedule. |
| Slings | Monthly and as required (or as manufacturer’s instructions) | - As possible: dedicate sling to client - All slings are numbered and weekly washed in accordance with their number - Care HCW send the slings to the laundry on the allocated day of washing - Laundry HCW mark that they have washed according to the sling number- that is on their monthly list - Use manufacturer washing instructions |
| Walking Frames | After each use and daily (weekly if single user) | - Use detergent wipes/disinfectant wipes - Wipe over all parts of the frames including handles and legs Include in routine cleaning schedule. |
| Wheelchairs | After each use and daily (weekly if single user) | - Use detergent wipes/disinfectant wipes - Wipe base and back of chair - Wipe underside of seat - Wipe handle grips - Wipe wheels and foot plates Include in routine cleaning schedule |
| Weigh Chairs | After each use and daily | - Use detergent wipes/disinfectant wipes - Wipe base and back of chair - Wipe underside of seat and bars - Wipe handle grips - Wipe wheels and foot plates Include in routine cleaning schedule |
| Syringe driver | Daily when in use | - Use detergent wipes/disinfectant wipes - Wipe over all surfaces and buttons - Wipe over any cords - Check the syringe driver as per instructions in the kit Include in routine cleaning schedule |
| General equipment | Frequency | Procedure |
| Linen skip frames | After use and daily | - Use detergent wipes/disinfectant wipes - Wipe the lid - Wipe the entire frame - Wipe the base and wheels Include in routine cleaning schedule. |
| Medication Trolley | After use and daily | - Use detergent wipes/disinfectant wipes - Wipe over all external surfaces - Take items out of draws and wipe inside draws and compartments - Wipe over wheels - Ensure there is wipes and hand gel on the trolley Include in routine cleaning schedule. |
| Dressing trolley | Before and after use | - Use disinfectant wipes - Wipe over top and bottom platform - Wipe frame around platforms - Wipe trolley legs - Wipe wheels Remove draws No items to be stored on the bottom shelf |
| Medication / Specimen fridges | Monthly | - Use detergent wipes/disinfectant wipes - Remove all items out of fridge – discard any out-of-date items - Wipe all racks and shelves - Wipe containers - Wipe internal walls, ceiling and floor - Wipe external walls top and door - Defrost the freezer and manage if required Include in routine cleaning schedule. |
| Tech equipment | Frequency | Procedure |
| Desk and computers on wheels laptops and keyboards | After use | - Use detergent wipes/disinfectant wipes - Wipe over all surfaces and buttons - Wipe over any cords and attachment - Check the device cleaning instructions |
| Printer | After use and full weekly | - Use detergent wipes/disinfectant wipes - Wipe over all surfaces and buttons - Wipe over any cords and holders - Check the device cleaning instructions Include in routine cleaning schedule |
| Single user equipment | Frequency | Procedure |
| Nebulizers | After every use and daily | - Single older person use - Use detergent wipes/disinfectant wipes - Wipe over the entire surface - Wipe under covers and in storage parts - Wipe over tubing and mask - Change tubing and mask weekly (or as manufacturer’s instructions) - Include in cleaning schedule. |
| Concentrator and nasal prongs | After each use and full monthly | - Single older person use - Use detergent wipes/disinfectant wipes - Wipe over the entire surface - Wipe under covers and in storage parts - Wipe over tubing and prongs - Change tubing and prongs weekly Include in cleaning schedule. |
| CPAP | After each use and full monthly | - Single older person use - Use detergent wipes/disinfectant wipes - CPAP Cleaning process (as per manufacturer’s instructions) Include in cleaning schedule. |
| Denture cup and toothbrush holder | After each use | - Single older person use - HCW to wear precaution PPE -apron and gloves - Clean mugs with warm water and detergent after each use, rinse - Store in older person’s bathroom |

# 4. References

- Australian Commission on Safety and Quality in Health Care. The aged care infection prevention and control guide [Internet]. Sydney: ACSQHC; 2024 [cited 2024 Nov 20]. Available from: https://www.safetyandquality.gov.au/sites/default/files/resources/attachments/The-Aged-Care-Infection-Prevention-and-Control-Guide.pdf

- National Health and Medical Research Council. Australian guidelines for the prevention and control of infection in health care [Internet]. Canberra: NHMRC; 2019 [cited 2024 Nov 20]. Available from: https://www.safetyandquality.gov.au/publications-and-resources/resource-library/australian-guidelines-prevention-and-control-infection-healthcare

- Standards Australia. AS 5369:2023 Reprocessing of reusable medical devices and other devices in health and non-health related facilities. Sydney: Standards Australia; 2023. Available from: https://www.standardsau.com/standards/as-5369-2023/

# 5. Endorsement / Approval

| Version | Date | Addition/Amendments | Author | Review By |
| --- | --- | --- | --- | --- |
| 1.0 | June 2024 | New resource | C Spinks IPC CNC | Aged Care IPC CNC group |
| 2.0 | Dec 2025 | Addition of non-medical equipment | C Spinks IPC CNC | Aged Care IPC CNC group, Terry McAuley (Medical Device Decontamination expert) |
| 3.0 | Jun 2026 | Branded and format change | C Spinks IPC CNC | Aged Care working Group |

# 6. Contact Information

ACIPC office: office@acipc.org.au